Shining 3D EinStar SDK User Manual

V1.5.0

Shining 3D Tech Co., Ltd.

| Revision History |  |  |
| --- | --- | --- |
| Version | Date | Revision Content |
| V1.0.0.0 | 2022-9-20 | First release. |
| V1.0.0.1 | 2023-3-6 | New sdk interface |
| V1.0.0.2 | 2023-3-13 | Add calibration tool |
| V1.0.0.3 | 2023-3-17 | Add mesh data interface |
| V1.0.0.4 | 2023-3-19 | Add device button event interface |
| V1.0.0.5 | 2023-3-30 | 1)Add get brightness interface.2)Add set work range interface.3)Repair get brightness range failed.4)Remove longpress_plus and longpress_sub funtion. |
| V1.0.0.6 | 20230713 | - Delete marker align type . - Delete auto align type. - Delete turtable align type. - Delete code points align type. - Add plane check interface. - Add get current marker interface. - Add get whole marker interface. |

1 SDK Description	3

1.1 Development Related Files	3

SDK List	3

Operation Requirements	3

1.2 Main Software Components	3

List of Documents	3

Operation Requirements	4

1.3 SDK Header File Description	4

1.4 Using the SDK Demo	4

1.5 SDK Known Issues	6

2 Data Structures and Macro Definitions	7

SN3D_IMAGE_DATA	7

SN3D_POINT_DATA	7

SN3D_INCREASE_POINT_CLOUD	8

SN3D_CAMERA_POSITION	9

SN3D_TRI_FACE	10

SN3D_TRI_MESH	11

SN3D_CLOUD_POINT	12

3 Return Value Definition	13

4 Alignment Type	14

5 Callback Function Declaration	15

Sn3DScanServiceWatcherCallBack	15

Sn3DWholePointCloudCallBack	16

Sn3DCurrentPointCloudCallBack	16

Sn3DIncreasePointCloudCallBack	16

Sn3DCameraPositionCallBack	17

Sn3DTrackLostStatusCallBack	17

Sn3DScanDistCallBack	18

6 Workflow	19

7 API Definition	20

7.1 Base Function Interface	20

Sn3DInitialize	20

Sn3DRelease	21

Sn3DInitialLog	22

Sn3DSaveData	23

7.2 Device Control Interface	24

Sn3DCheckDevice	24

Sn3DGetBrightnessRange	25

Sn3DSetBrightness	26

Sn3DIntallGetImagasCallBack	27

Sn3DSetCurrentPointCloudCallBack	27

Sn3DSetIncreasePointCloudCallBack	27

Sn3DSetCameraPositionCallBack	28

Sn3DSetTrackLostStatusCallBack	28

Sn3DSetScanDistCallBack	28

7.3 The Function Interface of Scan	29

Sn3DEnterScan	29

Sn3DOpenOrCreateSolution	30

Sn3DNewProject	30

Sn3DOpenProject	31

Sn3DPreviewScan	32

Sn3DStartScan	33

Sn3DPauseScan	34

Sn3DEndScan	35

Sn3DCleanScan	36

Sn3DExitScan	37

Sn3DSetEnablePseudoColor	38

7.4 Packaging Function Interface	39

Sn3DMesh	39

1 SDK Description

In the manual, EinStar is referred to as “the Scanner”, the software client is referred to as “the Client” and “scanservice.exe” is referred to as “the Service.”

1.1 Development Related Files

    - SDK List

| Item | Description |
| --- | --- |
| Shining 3D EinStar SDK User Manual | Software interfaces description manual. |
| Include | Header files for ZMQ development; API header files helping analyze shared memory. |
| Lib | Lib files For ZMQ development; API lib file helping analyze shared memory. |
| Bin | Software execution package. |
| ScannerDemo | SDK demo software source code. |
| calibration_tool | Calibration tool |

    - Operation Requirements

1, Use vs2017+Qt5.12.8

2, The output of ScannerDemo.exe after compilation is in the bin file (the demo install will copy it).

3, Copy zmq_sdk_config.ini to the working directory and modify isRemote=1 (demo install will copy it).

4, Copy the sn3d_smc.dll output in the bin file (the demo install will copy it).

5, Copy the sn3de10sdk.dll output in the bin file (the demo install will copy it).

1.2 Main Software Components

    - List of Documents

| Item | Name | Description |
| --- | --- | --- |
| Bin\zmq_sdk_config.ini | SDK configuration file | isRemote: 0 does not support SDK access 1 Support SDK access Pub: public messaging port, Default value 11398 Rep: Request messaging port Default value 11399 |
| Bin\scanservice.exe | Scan service | Scans and processes data. |
| ScannerDemo.exe | Scan client demo | Call scanner service to scan. |
| Bin\sn3dE10SDK.dll | SDK link library |  |

    - Operation Requirements

Modify interface in zmq_sdk_config.ini when communication port 11398 or 11399 is used by other software in your PC.

1.3 SDK Header File Description

| File Name | Description |
| --- | --- |
| Include\\Sn3DE10Sdk.h | Contains sdk interfaces declaration |
| Include\\Sn3DErrorCode.h | Contains error codes information |
| Include\\Sn3DPublic.h | Custom structure included |
| Include\\PublicClass.h | Public definitions |

1.4 Using the SDK Demo

1, Install CMake3.17.2 and above, install QT version 5.12.8;

2, Build solutions using cmake, Visual Studio 15 2017 + QT 5.12.8;

3, vs 2017 builds solutions under release x64

4, Copy ScannerDemo.exe, zmq_sdk_config.ini and all dynamic libraries under the folder bin to the root directory of the EinScan release software

5, Execute scannerDemo.exe under the root directory of release software.

6, Execute connectService (sdk initialization) and connectdevice (connection device) in turn.

7, Go to the scanner tab page, click start (start scan) and end (generate data).

Using the calibration tool

1) Copy calibration_tool.exe to the distribution software directory;

2) Connect the device, start it directly, and calibrate according to the prompt;

3) After the calibration calculation is completed, the calibration file will be generated in the directory res/Einscan-E10/200x150;

Note: When starting the calibration program, ensure that the ple file exists and the device is connected normally, otherwise it cannot be calibrated normally.

1.5 SDK Known Issues

- Garbled code for new construction projects under the Chinese path.

2 Data Structures and Macro Definitions

SN3D_IMAGE_DATA

Data structure

Image data

typedef struct tag SN3D_IMAGE_DATA { int width; int height; int channel; int length; unsigned char* data; } SN3D_ IMAGE_ DATA, *LPSN3D_ IMAGE_ DATA;

Members

width

The width of the image;

height

The height of the image;

channel

The number of image channels;

length

image data length = width*height;

data

Image data.

Remarks

SN3D_POINT_DATA

Data structure

Point data

typedef struct { float x; //X coordinate value float y;//Y coordinate value float z;//Z coordinate value }Sn3dPointData, *LPSn3dPointData;

Members

x

X coordinate.

y

Y coordinate.

z

Z coordinate.

Remarks

SN3D_INCREASE_POINT_CLOUD

Data structure

Point data

typedef struct { int vertex_count; //The number of vertex . LPSn3dPointData vertex_data; //The data of vertex. int norma_count; //The number of vertex normal. LPSn3dPointData norma_data; //The data of vertex normal. int vertex_color_count; //The number of vertex color. LPSn3dPointData vertex_color_data; //The data of vertex color. int index_count; //The number of index unsigned int* index; //The data of index }Sn3DIncreasePointCloud,*LPSn3dIncreasePointCloud;

Members

vertex_count

Vertices number

vertex_data

Vertices data

norma_count

Normal number

norma_data

Normal data

vertex_ color _count

Vertices color number

vertex_color_data

Vertices color data

index_count

The index number of point

index

The index data of point

Remarks

SN3D_CAMERA_POSITION

typedef struct { QVector3D position; QVector3D center; QVector3D up; }Sn3DCameraPosition,*LPSn3dCameraPosition;

Members

position

Camera Position

center

Center of view

up

Normal of up

Remarks

SN3D_FACE_ID

Triangular patch information

typedef struct { int x; int y; int z; }Sn3DFaceId, *LPSn3dFaceId;

Members

x, y, z: index of face

Remarks

SN3D_VEC2F

- typedef struct - { - float x; - float y; - }Sn3DVec2F, *LPSn3dVec2F;

Members

x, y: coordinate of texture

Remarks

SN3D_IMAGE

- typedef struct - { - int width; - int height; - int channel; - uchar* data; - //RGB - }Sn3DImage;

Members

width: width of image

height: height of image

channel: channel of image

data: data of image

Remarks

SN3D_MESHDATA

- typedef struct - { - int meshpoint_count; - LPSn3dPointData meshpoint; - int meshnormal_count; - LPSn3dPointData meshnormal; - int meshtrifaceid_count; - LPSn3dFaceId meshtrifaceid; - int meshtextureid_count; - LPSn3dId meshtextureid; - int textureUV_count; - LPSn3dVec2F textureUV; - Sn3DImage image; - }Sn3DMeshData, *LPSn3dMeshData

Members

meshpoint_coun: num of point cloud

meshpoint: data of point cloud

meshnormal_count: num of normal

meshnormal: 	data of normal

meshtrifaceid_count: num of point index

meshtrifaceid: data of point index

meshtextureid_count: num of textureimage

meshtextureid: data of texture image

- textureUV_count: num of uv

- textureUV:data of uv

image: texture image

Remarks

SN3D_CLOUD_POINT

Data structure

Point cloud data

typedef struct { int vertex_count; //The number of vertex . LPSn3dPointData vertex_data; //The data of vertex. int norma_count; //The number of vertex normal. LPSn3dPointData norma_data; //The data of vertex normal. int vertex_color_count; //The number of vertex color. LPSn3dPointData vertex_color_data; //The data of vertex color. }Sn3DPointCloud,*LPSn3dPointCloud;

Members

vertex_count

Number of vertex data.

vertex_data

Vertex data.

norma_count

Number of vertex normal data.

norma_data

Vertex normal data.

vertex_ color _count

The number of vertex color data.

vertex_color_data

Vertex color data.

Remarks

SN3D_MARKER_DATA

- struct MarkerInfo - { - Sn3dPointData coord; // data of marker - Sn3dPointData normal; // data of noraml - double diameter; // diameter - int codeId; //marker id (<=0 means common marker) - }; - typedef struct { - int marker_count; //marker count - MarkerInfo* markerList; //marker list - }Sn3DMarkerData, *LPSn3DMakerData;

Members

Information of single marker：

coord:       marker data.

normal:		marker normal data.

diameter:		marker diameter.

codeId:		marker id (<=0 means common marker).

Information of marker list:

marker_count: number of markers.

markerList:	list of markers.

Remarks

3 Return Value Definition

| Macro definition | Value | Description |
| --- | --- | --- |
| EC_SUCCESS | 0 | No error |
| EC_NOTINITIALIZED | 1 | No initialization |
| EC_INITIALIZEFIAILZED | 2 | Initialization failed. |
| EC_AlREADYINITIALIZED | 3 | Initialization repeat. |
| EC_SAVEFAILED_TYPEERROR | 4 | Format error，failed to save files. |
| EC_CHECKDEVICEFAILED_PLENOTRIGHT | 5 | PLE error, failed to connect device. |
| EC_CHECKDEVICEFAILED_NODEVICEFOUND | 6 | Device no found, connection failed. |
| EC_OPENORCREATSLNFAILED | 7 | Failed to create or open project. |
| EC_ENTERSCANFAILED | 8 | Failed to enter scan mode. |
| EC_SCANFAILED | 9 | Scanning failed. |
| EC_ENDSCANFAILED | 10 | Generating failed. |
| EC_MESHFAILED | 11 | Meshing failed. |
| EC_EXITSCANFAILED | 12 | Quitting scan mode failed. |
| EC_CANCELSCANFAILED | 13 | Canceling scan failed. |
| EC_SAVEFAILED | 14 | Saving failed. |
| EC_CREATNEWPROJECTFAILED | 16 | New project failed. |
| EC_WORKRANGE | 17 | Set faied |
| EC_PROCESSERROR | 268435456 | Call process error 0x10000000 |

4 Alignment Type

| Type definition | Value | Description |
| --- | --- | --- |
| AT_FEATURES | 0 | Feature alignment |
| AT_HYBRID | 2 | Hybrid alignment |
| AT_GLOBLE_POINT | 7 | Global markers alignment |
| AT_TEXTURE | 8 | Texture alignment |

5 DeviceEvent Type

| 枚举 | 值 | 含义 |
| --- | --- | --- |
| DE_NULL | -1 | NULL |
| DE_DOUBLECLICK | 0 | Double Click |
| DE_CLICK | 1 | Click |
| DE_PLUS | 2 | Plus button click |
| DE_SUB | 3 | Sub button click |
| DE_LONGPRESS | 8 | Ok button long press |

5 Callback Function Declaration

Sn3DScanServiceWatcherCallBack

Initialize the callback

Initialize the device setup callback.

typedef void (CALLBACK *Sn3DScanServiceWatcherCallBack)(int);

Remarks

Register the server monitors the callback and the server is notified of the exit.

Sn3DWholePointCloudCallBack

Point cloud callback

Initialize the device setup callback

typedef void(CALLBACK *Sn3DWholePointCloudCallBack)(LPSn3dPointCloud wholePointCloud);

Parameter

wholePointCloud

[out] point cloud

Remarks

1, Setting sn3dEndscan (end scan) or sn3dMesh (meshing) callback can get point cloud set at different stages.

2, The callback needs to be reset after setting a callback is successfully triggered.

Sn3DCurrentPointCloudCallBack

Get current point cloud callback

typedef void(CALLBACK *Sn3DCurrentPointCloudCallBack)(LPSn3dPointCloud currentPointCloud, void* owner);

Parameter

currentPointCloud

[out] current point cloud

owner

[out] the owner of callback function

Remarks

- 1.Setting callback after Sn3DInitialize(initial) will get the point convergence of the current frame throughout the scan.

Sn3DIncreasePointCloudCallBack

Get increase point cloud callback funtion

typedef void(CALLBACK *Sn3DIncreasePointCloudCallBack)(LPSn3dIncreasePointCloud increasePointCloud, void* owner);

Parameter

increasePointCloud

[out] increase point cloud data

owner

[out] the owner of callback function

Remarks

- 1.Setting callback after Sn3DInitialize(initial) will get the point convergence of the increase point throughout the scan。

Sn3DCameraPositionCallBack

Get Camera Position callback function

typedef void(CALLBACK *Sn3DCameraPositionCallBack)(LPSn3dCameraPosition cameraPosition, void* owner);

Parameter

cameraPosition

[out] The data of camrea position

owner

[out] the owner of callback function

Remarks

- 1.Setting callback after Sn3DInitialize(initial) will get the point convergence of the camera position throughout the scan

Sn3DTrackLostStatusCallBack

Get track lost status callback funtion

typedef void(CALLBACK *Sn3DTrackLostStatusCallBack)(bool trackLostStatus, void* owner);

Parameter

trackLostStatus

[out] The status of track，true: Track lost；false: track success

owner

[out] the owner of callback function

Remarks

- 1.Setting callback after Sn3DInitialize(initial) will get the point convergence of the track lost status  throughout the scan

Sn3DScanDistCallBack

Get scan distance gear callback funtion

typedef void(CALLBACK *Sn3DScanDistCallBack)(double scanDist, void* owner);

Parameter

scanDist

[out] The distance gear of scan

owner

[out] the owner of callback function

Remarks

1.Setting callback after Sn3DInitialize(initial) will get the point convergence of the distance gear  throughout the scan

2.scanDist = -1:too near；scanDist = 100:too far；scanDist=-2，invalid；other (1~12)。

Sn3DMeshDataCallBack

typedef void(CALLBACK *Sn3DMeshDataCallBack)(LPSn3dMeshData meshData, void* owner);

Parameter

meshData

[out] mesh of data

owner

[out] the owner of callback function

Remarks

.Setting callback after Sn3DEndScan will get the point convergence of the track lost status  throughout the scan

Sn3DDeviceEventCallBack

typedef void(CALLBACK *Sn3DDeviceEventCallBack)(DeviceEvent event, void* owner);

Parameter

event

[out] type of button click

owner

[out] the owner of callback function

Remarks

- Setting callback after Sn3DInitialize(initial).

Sn3DTooFlatStatusCallBack

typedef void(CALLBACK *Sn3DTooFlatStatusCallBack)(bool tooFlatStatus, void* owner);

Parameter

tooFlatStatus

[out] check plane. The output will be true when object is plane.

owner

[out] the owner of callback function

Remarks

Sn3DCurrentMarkerCallBack

typedef void(CALLBACK *Sn3DCurrentMarkerCallBack)(LPSn3DMakerData currentMarker, void* owner);

Parameter

currentMarker

[out] current markers list.

owner

[out] the owner of callback function

Remarks

Sn3DWholeMarkerCallBack

typedef void(CALLBACK *Sn3DWholeMarkerCallBack)(LPSn3DMakerData wholeMarker, void* owner);

Parameter

wholeMarker

[out] whole markers list.

owner

[out] the owner of callback function

Remarks

6 Workflow

7 API Definition

7.1 Base Function Interface

Sn3DInitialize

Initialization SDK

void* Sn3DInitialize( Sn3DScanServiceWatctherCallBack watcher );

Parameters

Sn3DScanServiceWatctherCallBack

[in]  Registration service monitors the callback and receives notice when the server ended.

Return Values

For macro definition, refer to return values.

Remarks

1, Just initialize SDK once.

2, After initializing SDK, you can call Sn3DRelease to release.

Sn3DRelease

Release SDK

int Sn3DRelease();

Return Values

For macro definition, refer to return values.

Remarks

The function for release the SDK resource.

Sn3DInitialLog

Set up the SDK output log.

int Sn3DInitialLog ( char* exeName, char* path );

Parameters

exeName

[in]   Call name of SDK proceeding.

path

[in]   Set up directory of log in.

Return Values

Refer to error code.

Sn3DSaveData

Save data

int Sn3DSaveData ( char* absolutePrjName, char*savePath, char*saveType );

Parameters

absolutePrjName

[in] Complete path of project files.

savePath

[in]   Path and name of the file required to be saved (without a suffix).

saveType

[in] The format of the file required to be saved (asc ply stl obj).

Return Values

Refer to error code.

7.2 Device Control Interface

Sn3DCheckDevice

Connection device.

int Sn3DConnectDevice ()

Return Values

For macro definition, refer to return values.

Sn3DGetBrightnessRange

The brightness range

SN3DSDKE10API int Sn3DGetBrightnessRange(int& min, int& max);

Parameters

min

[out]   Reserved, the minimum brightness level is set to 0 (default). By default, this function is disable.

[out]   Maximum level of brightness.

Return Values

Refer to error code for details.

Remarks

Sn3DSetBrightness

Set the brightness

int Sn3DSetBrightness ( int brightness )

Parameters

brightness

[in] Brightness level, the value range is determined by the value returned by Sn3DGetBrightnessRange,

the larger the value, the brighter.

Return Values

Refer to error code.

Remarks

Sn3DIntallGetImagasCallBack

Get video stream

int Sn3DIntallGetImagasCallBack ( Sn3DGetImagesCallBack imageCallback, void* owner, );

Parameters

imageCallback

[in]  Set the callback function.

owner

[in]  Pointer to the owner of the callback function.

Sn3DSetCurrentPointCloudCallBack

int Sn3DSetCurrentPointCloudCallBack ( Sn3DCurrentPointCloudCallBack callBackFunc, void* owner, );

Parameters

CallbackFunc

[in] Set callback funtion

owner

[in]  The owner of funciton

Sn3DSetIncreasePointCloudCallBack

int Sn3DSetIncreasePointCloudCallBack( Sn3DIncreasePointCloudCallBack callBackFunc, void* owner, );

Parameters

CallbackFunc

[in] Set callback funtion

owner

[in]  The owner of funciton

Sn3DSetCameraPositionCallBack

int Sn3DSetCameraPositionCallBack( Sn3DCameraPositionCallBack callBackFunc, void* owner, );

Parameters

[in] Set callback funtion

owner

[in]  The owner of funciton

Sn3DSetTrackLostStatusCallBack

int Sn3DSetTrackLostStatusCallBack( Sn3DTrackLostStatusCallBack callBackFunc, void* owner, );

Parameters

[in] Set callback funtion

owner

[in]  The owner of funciton

Sn3DSetScanDistCallBack

int Sn3DSetScanDistCallBack( Sn3DScanDistCallBack callBackFunc, void* owner, );

Parameters

[in] Set callback funtion

owner

[in]  The owner of funciton

Sn3DSetDeviceEventCallBack

int Sn3DSetDeviceEventCallBack( Sn3DDeviceEventCallBack callBackFunc, void* owner, );

Parameters

[in] Set callback funtion

owner

[in]  The owner of funciton

Sn3DSetWorkRange

int Sn3DSetWorkRange(float min, float max);

Parameters

min

[in] The minimum working distance，The defaut value is 160

max

[in] The maximum working distance，The defaut value is 600

Return Values

Error code

Remarks

Use this function after enter scan and before exit scan.

Sn3DGetCurrentBrightness

int Sn3DGetCurrentBrightness(int& currentBrightness)

Parameters

currentBrightness

[out] Current brightness value

Return Values

Error code

Remarks

Sn3DSetTooFlatStatusCallBack

int Sn3DSetTooFlatStatusCallBack(Sn3DTooFlatStatusCallBack callBackFunc, void* owner)；

Parameters

CallbackFunc

[in] Set callback function

owner

[in]  The owner of function

Sn3DSetCurrentMarkerCallBack

int Sn3DSetCurrentMarkerCallBack(Sn3DCurrentMarkerCallBack callBackFunc, void* owner)；

Parameters

CallbackFunc

[in] Set callback function

owner

[in]  The owner of function

Sn3DSetWholeMarkerCallBack

int Sn3DSetWholeMarkerCallBack(Sn3DWholeMarkerCallBack callBackFunc, void* owner)；

Parameters

CallbackFunc

[in] Set callback function

owner

[in]  The owner of function

7.3 The Function Interface of Scan

Sn3DEnterScan

Enter scan

SN3DSDKE10API int CALLMETHOD Sn3DEnterScan();

Parameters

Return Values

Error code, refer to macro definition of return values.

Remarks

1, This function can only be called after initializing the detection device online

2, Perform the function of initializing scan.

Sn3DOpenOrCreateSolution

New solutions

SN3DSDKE10API int Sn3DOpenOrCreateSolution( const char* slnDirPath, bool isCreate, int scanMode, bool hasTexture, float pointDis);

Parameters

slnDirPath

[in] Solution path

isCreate

[in] “true”- new, “false”-open

scanMode

[in] Scan mode, 0 portrait, 1 object, 2 smallobject. Reference: ScanMode

hasTexture

[in] “true”- texture, “false”-no texture

pointDis

[in] Point distance

Return Values

Error code refer to macro definition of return values.

Remarks

Call Sn3DOpenOrCreateSolution after Sn3DEnterScan.

Sn3DNewProject

New project

SN3DSDKE10API int CALLMETHOD Sn3DNewProject( const char* slnDirPath, int scanMode, bool hasTexture, float pointDis, int alignType, const char* globalMarkerPath);

Parameters

slnDirPath

[in] Solution Path

scanMode

[in] Scan mode, 0 portrait, 1 object, 2 smallobject. Reference: ScanMode

hasTexture

[in]  “true”- texture, “false”-no texture

pointDis

[in] Point distance

alignType

[in] Alignment type. Reference: AlignType  See appendix

globalMarkerPath

[in] Global markers path，no global markers path：set scan mode to empty.

Return Values

Error code refer to macro definition of return values.

Remarks

Call this function after call Sn3DOpenOrCreateSolution.

Sn3DOpenProject

Open project

SN3DSDKE10API int CALLMETHOD Sn3DOpenProject( const char* projFileName);

Parameters

projFileName

[in] Project path

Return Values

Error code refer to macro definition of return values.

Remarks

Call this function after call Sn3DOpenOrCreateSolution

Sn3DPreviewScan

Preview

SN3DSDKE10API int CALLMETHOD Sn3DPreviewScan();

Parameters

Return Values

Error code refer to macro definition of return values.

Remarks

1, Preview only rebuild the data not save the scan data to overall point cloud.

2, Call this function after you create or open project.

Sn3DStartScan

Scan

SN3DSDKE10API int CALLMETHOD Sn3DStartScan();

Parameters

Return Values

Error code refer to macro definition of return values.

Remarks

1, Call this function after Sn3DPreviewScan.

2, Tracking, and fusion to the overall point cloud.

Sn3DPauseScan

Pause Scan

SN3DSDKE10API int CALLMETHOD Sn3DPauseScan ();

Parameters

Return Values

Error code refer to macro definition of return values.

Remarks

- Call this function after call Sn3DStartScan.

Sn3DEndScan

End scan

SN3DSDKE10API int CALLMETHOD Sn3DEndScan( bool globalOptimize, double pointDist, Sn3DWholePointCloudCallBack callback = nullptr);

Parameters

globalOptimize

[in] “true”-optimize , “false” -not optimized

pointDist

[in]setting point distance.

callback

[in]Set up the callback to get the point cloud data after finishing the scan.

Return Values

Error code refer to macro definition of return values.

Remarks

1, You can call Sn3DEndScan after Sn3DStartScan or Sn3DPauseScan.

2, Generate the overall point cloud data;

3, If you set the callback to get that stage to generate point cloud data, there are examples in demo.

Sn3DCleanScan

Empty the scan data

SN3DSDKE10API int Sn3DCancelScan( bool isCancelCurrentProjectFramerMark);

Parameters

isCancelCurrentProjectFramerMark

[in] “true”- Empty the global markers information.

Return Values

Error code refer to macro definition of return values.

Remarks

- Empty the currently scanned point cloud data.

Sn3DExitScan

Quit scan mode

SN3DSDKE10API int CALLMETHOD Sn3DExitScan();

Parameters

Return Values

Error code refer to macro definition of return values.

Remarks

- Quit scan mode.

Sn3DSetEnablePseudoColor

Turn on pseudo function

int Sn3DSetEnablePseudoColor( bool enable );

Parameters

enable

[in]  “true”-open, “false”-off

Return Values

Refer to error code.

7.4 Packaging Function Interface

Sn3DMesh

Generate mesh data

SN3DSDKE10API int CALLMETHOD Sn3DMesh( int meshType, int filterLevel, int SmoothLevel, float PointDis, bool fillSmallHole, double smellHolePerimeter, bool fillMarkerHole, double spikeSensitivity, int faceLimit, Sn3DWholePointCloudCallBack callback = nullptr);

Parameters

meshType.

[in] Reference: MeshType

filterLevel.

[in] 0-3 level, different levels of filtering degree.

SmoothLevel.

[in] 1-3 level, Setting up smooth level in filter.

PointDis.

[in]point distance.

fillSmallHole.

[in] “true”-Fill the small hole

smellHolePerimeter.

[in]Fill the circumference of the small hole

fillMarkerHole.

[in] “true”-filling markers.

spikeSensitivity.

[in] Remove the spikes

faceLimit.

[in] Maximum number of patch.

callback.

[in] setting up callback for getting point cloud data of meshed.

Return Values

Refer to error code.

Remarks

1, Calling Sn3DEndScan after ending scan.

2, Meshing data.

3, Setting up callback for getting overall point cloud set of meshed.

Sn3DMeshEx

SN3DSDKE10API int CALLMETHOD Sn3DMesh( int meshType, int filterLevel, int SmoothLevel, float PointDis, bool fillSmallHole, double smellHolePerimeter, bool fillMarkerHole, double spikeSensitivity, int faceLimit, void* owner Sn3DMeshDataCallBack callback = nullptr);

Parameters

meshType.

[in] Reference: MeshType

filterLevel.

[in] 0-3 level, different levels of filtering degree.

SmoothLevel.

[in] 1-3 level, Setting up smooth level in filter.

PointDis.

[in]point distance.

fillSmallHole.

[in] “true”-Fill the small hole

smellHolePerimeter.

[in]Fill the circumference of the small hole

fillMarkerHole.

[in] “true”-filling markers.

spikeSensitivity.

[in] Remove the spikes

faceLimit.

[in] Maximum number of patch.

Owner

[in] owner of callback

callback.

[in] setting up callback for getting mesh data of meshed.

Return Values

Refer to error code.

Remarks

1, Calling Sn3DEndScan after ending scan.

2, Meshing data.

3, Setting up callback for getting overall point cloud set of meshed.